| 1. IDENTIFICATION OF THE MATERIAL AND SUPPLIER |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Product name: |  |  |  | Sabakem Imazapic 240SL Herbicide |  |  |  |  |  |
| Other means of identification: |  |  |  | Soluble concentrate containing Imazapic |  |  |  |  |  |
| Recommended use of the chemical and restrictions on use: |  |  |  | Agricultural Herbicide for use as described on the product label. |  |  |  |  |  |
| Supplier: |  |  |  | Sabakem Pty Ltd |  |  |  |  |  |
| Street address: |  |  |  | Suite 809, Level 8, 2 Queen St Melbourne VIC 3000 Australia |  |  |  |  |  |
| Telephone no.: |  |  |  | 03 9629 3979 |  |  |  |  |  |
| Website: |  |  |  | www.sabakem.com |  |  |  |  |  |
| Emergency telephone: |  |  |  | Poisons Information Centre 13 11 26 (24 hours) |  |  |  |  |  |
| 2. HAZARDS IDENTIFICATION |  |  |  |  |  |  |  |  |  |
| Classification of the substance mixture: |  |  |  | This material is hazardous according to Globally Harmonised System of Classification and labelling of Chemicals (GHS) including Work, Health and Safety regulations, Australia. Classification of the substance or mixture: Eye Damage/irritation – Category 2B The following environment hazard categories fall outside the scope of the Workplace Health and Safety Regulations: Aquatic acute toxicity – Category 1 chronic toxicity – Category 1 SIGNAL WORD: WARNING Hazard Statement(s): H320 – Causes eye irritation H400 – Very toxic to aquatic life H410 – Very toxic to aquatic life with long lasting effects. Precautionary Statement(s): Prevention: P264 – Wash contacted areas thoroughly after handling. P273 – Avoid release to the environment Response: P305 + P351 + P338 – IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. P337 + P313 – If eye irritation occurs: Get medical advice/attention. P391 – Collect spillage Disposal: P501 - Dispose of contents/container as per container label, in accordance with local/state/territory government regulations. |  |  |  |  |  |
| 3. COMPOSITION/INFORMATION ON INGREDIENTS |  |  |  |  |  |  |  |  |  |
| Components |  |  |  |  |  |  | CAS Number |  | Proportion (w/v) |
| Imazapic, present as ammonium salt |  |  |  |  |  |  | 104098-48-8 |  | 240 g/L |
| Other components are not considered hazardous in this formulation or does not cause the correct hazard classification and therefore are not required to be disclosed according to the WHS Regulations. |  |  |  |  |  |  |  |  |  |
| 4. FIRST AID MEASURES |  |  |  |  |  |  |  |  |  |
| Speed in treatment is essential. If poisoning occurs, contact a Poisons Information Centre. Phone Australia 131126; New Zealand 0800 764 766 or a doctor. Have this SDS or the label with you. |  |  |  |  |  |  |  |  |  |
| Inhalation: |  | No first aid measures normally required. However if inhaled, bring affected person to fresh air. If breathing is difficult, give oxygen. If symptoms develop, contact a Poisons Information Centre or a doctor at once. |  |  |  |  |  |  |  |
| Skin contact: |  | If on skin, remove contaminated clothing and wash with plenty of water and soap. If symptoms develop, seek medical attention. |  |  |  |  |  |  |  |
| Eye contact: |  | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. Continue flushing until advised to stop by a Poisons Information Centre or a doctor, or for at least 15 minutes. Seek immediate medical advice. |  |  |  |  |  |  |  |
| Ingestion: |  | If swallowed, rinse mouth and contact a Poisons Information Centre, or call a doctor if you feel unwell. Do not induce vomiting unless told to do so by the Poisons Information Centre or doctor. |  |  |  |  |  |  |  |
| First aid facilities: |  | Eyewash and normal washroom facilities. |  |  |  |  |  |  |  |
| Medical attention and special treatment: |  | Treat symptomatically. |  |  |  |  |  |  |  |
| 5. FIRE FIGHTING MEASURES |  |  |  |  |  |  |  |  |  |
| Suitable extinguishing media: |  |  | Not Combustible. Use extinguishing media suited to burning materials. |  |  |  |  |  |  |
| Specific hazards arising from the substance or mixture: |  |  | There is no risk of an explosion from this product under normal circumstances if it is involved in a fire. Only small quantities of decomposition products are expected from this product at temperatures normally achieved in a fire. This will only occur after heating to dryness. Fire decomposition products from this product may be toxic if inhaled. Take appropriate protective measures. |  |  |  |  |  |  |
| Special protective equipment and precautions for fire-fighters: |  |  | In case of fire and/or explosion do not breathe fumes. Wear self-contained breathing apparatus and chemical-protective clothing. Keep containers cool by spraying with water if exposed to fire. Collect contaminated extinguishing water separately. Do not allow contaminated water to reach sewage or effluent systems. Dispose of fire debris and contaminated extinguishing water in accordance with official regulations. |  |  |  |  |  |  |
| Hazchem code: |  |  | •3Z (bulk only) |  |  |  |  |  |  |
| 6. ACCIDENTAL RELEASE MEASURES |  |  |  |  |  |  |  |  |  |
| Emergency procedures/ Environmental precautions: |  |  | In the event of a spill, prevent spillage from entering drains or water courses with absorbent material and call emergency services. |  |  |  |  |  |  |
| Personal precautions/ Protective equipment: |  |  | Wear appropriate personal protective equipment and clothing to prevent exposure. Provide adequate ventilation. Evacuate all non-essential personnel from affected area. Do not breathe vapours. Wear protective clothing as described in Section 8 of this safety data sheet. |  |  |  |  |  |  |
| Methods and materials for containment and cleaning up: |  |  | Contain - prevent run off into drains and waterways. For minor spills, clean up, rinsing to sewer and put empty container in garbage. Label the containers containing waste and contaminated materials and remove from the area as soon as possible. Flush contaminated area with plenty of water. Wash thoroughly after dealing with a spillage. |  |  |  |  |  |  |
| 7. HANDLING AND STORAGE |  |  |  |  |  |  |  |  |  |
| Precautions for safe handling: |  |  | Keep exposure to this product to a minimum, and minimise the quantities kept in work areas. Keep containers closed at all times - check regularly for leaks or spills. Transport and store upright. Refer to Section 8 of this SDS for details of personal protective measures, and make sure that those measures are followed. The measures detailed below under ‘Storage’ should be followed during handling in order to minimise risks to persons using the product in the workplace. Also, avoid contact or contamination of the product with incompatible materials listed in Section 10. |  |  |  |  |  |  |
| Conditions for safe storage, including any incompatibilities: |  |  | Store in the closed, original container in a cool, well-ventilated area out of direct sunlight. Keep container tightly sealed. Make sure that the product does not come into contact with substances listed under ‘Incompatibilities’ in Section 10. Check packaging - there may be further storage instructions on the label. |  |  |  |  |  |  |
| 8. EXPOSURE CONTROLS/PERSONAL PROTECTION |  |  |  |  |  |  |  |  |  |
| Exposure control parameters: |  |  | No value assigned for this specific material by Safe Work Australia. No biological limit allocated for the product or any of its ingredients. No biological monitoring is required. |  |  |  |  |  |  |
| Engineering controls: |  |  | Use in well-ventilated areas. Keep containers closed when not in use. |  |  |  |  |  |  |
| Individual protection measures, such as Personal Protective Equipment (PPE): |  |  |  |  |  |  |  |  |  |
| See container label safety directions. The selection of PPE is dependent on a detailed risk assessment. The risk assessment should consider the work situation, the physical form of the chemical, the handling methods, and environmental factors. Observe good standards of hygiene and cleanliness. Always wash hands, arms and face thoroughly with soap and water before smoking, eating, drinking or using the toilet. Wash contaminated clothing and other protective equipment with detergent and warm water before storage or re-use. |  |  |  |  |  |  |  |  |  |
| Respiratory protection: |  |  | If engineering controls are not effective in controlling airborne exposure, then an approved respirator should be used. See Australian Standards AS/NZS 1715 and AS/NZS 1716 for more information. |  |  |  |  |  |  |
| Eye and face protection: |  |  | Avoid contact with eyes. Eye protection such as protective glasses or goggles must be worn when product is being used. Failure to protect your eyes may cause them harm. Emergency eye wash facilities are also recommended in an area close to where this product is being used. Consult AS/NZS 1336 and AS/NZS 1337 for further information. |  |  |  |  |  |  |
| Skin protection: |  |  | Full protective clothing, and elbow-length rubber or chemical resistant gloves should be worn when opening the container and using the product. Always check with the glove manufacturer or your personal protective equipment supplier regarding the correct type of glove to use. Consult AS/NZS 2161, AS/NZS 4501 and AS/NZS2210 for further information. |  |  |  |  |  |  |
| 9. PHYSICAL AND CHEMICAL PROPERTIES |  |  |  |  |  |  |  |  |  |
| Physical state: |  |  |  |  |  | Liquid |  |  |  |
| Colour: |  |  |  |  |  | Clear amber to green brown |  |  |  |
| Odour: |  |  |  |  |  | Mild ammonia |  |  |  |
| pH: |  |  |  |  |  | 6.4 – 6.8 |  |  |  |
| Specific gravity: |  |  |  |  |  | 1.090 |  |  |  |
| Melting point/Freezing point: |  |  |  |  |  | Below 0°C. |  |  |  |
| Boiling point/range: |  |  |  |  |  | Approximately 100°C at 100kPa. |  |  |  |
| Flash point: |  |  |  |  |  | No information available |  |  |  |
| Evaporation point: |  |  |  |  |  | No information available |  |  |  |
| Vapour pressure: |  |  |  |  |  | 2.37 kPa at 20°C (water vapour pressure). |  |  |  |
| Vapour density: |  |  |  |  |  | No information available |  |  |  |
| Solubility: |  |  |  |  |  | Completely soluble in water. |  |  |  |
| Partition coefficient: n- octanol/water |  |  |  |  |  | 0.393 (log P octanol/water) |  |  |  |
| Auto-ignition temperature: |  |  |  |  |  | Not applicable - does not burn. |  |  |  |
| Decomposition temperature: |  |  |  |  |  | No information available |  |  |  |
| Viscosity: |  |  |  |  |  | No information available |  |  |  |
| 10. STABILITY AND REACTIVITY |  |  |  |  |  |  |  |  |  |
| Reactivity: |  |  |  |  | This product is unlikely to react or decompose under normal storage conditions. However, if you have any doubts, contact the supplier for advice on shelf life properties. |  |  |  |  |
| Chemical stability: |  |  |  |  | Stable under normal ambient and anticipated storage and handling conditions of temperature and pressure. |  |  |  |  |
| Possibility of hazardous reactions: |  |  |  |  | No information available. |  |  |  |  |
| Conditions to avoid: |  |  |  |  | Protect this product from light. Store in the closed original container in a dry, cool, well-ventilated area out of direct sunlight. |  |  |  |  |
| Incompatible materials: |  |  |  |  | Strong oxidizing agents, acids and bases. |  |  |  |  |
| Hazardous decomposition products: |  |  |  |  | Only small quantities of decomposition products are expected from this product at temperatures normally achieved in a fire. This will only occur after heating to dryness. Combustion forms carbon dioxide, and if incomplete, carbon monoxide and smoke. Water is also formed. May form nitrogen and its compounds, and under some circumstances, oxides of nitrogen. Occasionally hydrogen cyanide gas in reducing atmospheres. Carbon monoxide poisoning produces headache, weakness, nausea, dizziness, confusion, dimness of vision, disturbance of judgment, and unconsciousness followed by coma and death. |  |  |  |  |
| 11. TOXICOLOGICAL INFORMATION |  |  |  |  |  |  |  |  |  |
| Acute toxicity: |  |  | Not considered to be acutely toxic via oral, dermal or inhalation routes of exposure, according to available information. No acute toxicity data is available for the formulation. The toxicity data for the active constituent, Imazapic: Oral LD50 (rat): >5000 mg/kg Dermal LD50 (rat): > 2000 mg/kg Inhalation LC50 (rat): 4.83 mg/L/4hr |  |  |  |  |  |  |
| Skin irritation: |  |  | Not considered to be a skin irritant according to available information. |  |  |  |  |  |  |
| Eye irritation: |  |  | Causes serious eye irritation according to available information. |  |  |  |  |  |  |
| Respiratory or skin sensitisation: |  |  | Not a skin sensitiser and not expected to be a respiratory sensitiser according to available information. |  |  |  |  |  |  |
| Germ cell mutagenicity: |  |  | Not suspected to cause genetic defects according to available data. |  |  |  |  |  |  |
| Carcinogenicity: |  |  | Not considered to be carcinogenic according to available data. |  |  |  |  |  |  |
| Reproductive toxicity: |  |  | Not considered to be toxic to reproduction according to available data. |  |  |  |  |  |  |
| STOT-single exposure: |  |  | May cause respiratory irritation through single exposure according to available information. |  |  |  |  |  |  |
| STOT-repeated exposure: |  |  | Not expected to cause toxicity to a specific target organ according to available data. |  |  |  |  |  |  |
| Aspiration hazard: |  |  | Not expected to be an aspiration hazard according to available information. |  |  |  |  |  |  |
| Chronic health effects: |  |  | Not expected to cause chronic health effects according to available data. |  |  |  |  |  |  |
| 12. ECOLOGICAL INFORMATION |  |  |  |  |  |  |  |  |  |
| Ecotoxicity: |  |  |  | Considered to be toxic to aquatic life based on available information. No toxicological data is available for the formulation. The toxicity data for the active constituent, Imazapic (ISO): Lemna gibba 14-day NOEL = 2.58 µg/L (0.00258 mg/L) |  |  |  |  |  |
| Persistence/Degradability: |  |  |  | No information available on the product. mazapic is rapidly degraded in water/sunlight but may be persistent in soil. |  |  |  |  |  |
| Bioaccumulative potential: |  |  |  | No information available on the product. Imazapic is not expected to bioaccumulate. |  |  |  |  |  |
| Mobility in soil: |  |  |  | No information available on the product. Imazapic is moderately mobile. |  |  |  |  |  |
| 13. DISPOSAL CONSIDERATIONS |  |  |  |  |  |  |  |  |  |
| Disposal methods: |  |  | Refer to Waste Management Authority. Dispose of contents/container in accordance with local/regional/national/international regulations. Break, crush or puncture and dispose of empty containers in a local authority landfill. Triple rinse and bury rinsate and empty capsules in a local authority landfill. If no landfill is available, bury the containers below 0.5 m in a disposal pit specifically marked and set up for this purpose clear of waterways, desirable vegetation and tree roots. Empty containers and product must not be burnt. Do NOT re-use containers for any other purpose. |  |  |  |  |  |  |
| 14. TRANSPORT INFORMATION |  |  |  |  |  |  |  |  |  |
| Road and rail transport: | Environmentally Hazardous Substances meeting the descriptions of UN 3077 or UN 3082 are not subject to the provisions of the Australian Code for the Transport of Dangerous Goods by Road and Rail when transported by road or rail in; (a) packagings that do not incorporate a receptacle exceeding 500 kg(L); (b) or IBCs. |  |  |  |  |  |  |  |  |
| Marine transport: | Classified as Dangerous Goods by the criteria of the International Maritime Dangerous Goods Code (IMDG Code) for transport by sea; MARINE POLLUTANT |  |  |  |  |  |  |  |  |
|  | UN Number: |  |  |  |  |  |  | 3082 |  |
|  | Proper Shipping Name or Technical Name: |  |  |  |  |  |  | ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S. (CONTAINS IMAZAPIC) |  |
|  | Transport Hazard Class: |  |  |  |  |  |  | 9 |  |
|  | Packaging Group: |  |  |  |  |  |  | III |  |
|  | IMDG EMS Fire: |  |  |  |  |  |  | F – A |  |
|  | IMDG EMS Spill: |  |  |  |  |  |  | S - F |  |
|  | Environmental hazards: |  |  |  |  |  |  | Yes |  |
|  | Additional Information: |  |  |  |  |  |  | The marine pollutant mark is not required when transported in sizes of ≤ 5 L or ≤ 5 kg. |  |
| Air transport: | Not classified as Dangerous Goods by the criteria of the International Air Transport Association (IATA) Dangerous Goods Regulations for transport by air; NON- DANGEROUS GOODS. |  |  |  |  |  |  |  |  |
| 15. REGULATORY INFORMATION |  |  |  |  |  |  |  |  |  |
| Poison schedule (SUSMP): |  |  |  | Schedule 0 |  |  |  |  |  |
| APVMA approval no.: |  |  |  | 69276 |  |  |  |  |  |
| AICIS: |  |  |  | All the constituents of this material are either listed on the Australian Inventory of Industrial Chemicals (AIIC) or not required as they are excluded as an industrial chemical being regulated as Agricultural Chemical products as per the Agricultural and Veterinary Chemicals Code Act (1994). |  |  |  |  |  |
| 16. OTHER INFORMATION |  |  |  |  |  |  |  |  |  |
| General information: |  |  | None |  |  |  |  |  |  |
| Issue number: |  |  | July 2026 |  |  |  |  |  |  |
| Issue date: |  |  | 06 July 2026 |  |  |  |  |  |  |
| In any event, the review and, if necessary, the re-issue of an SDS shall be no longer than 5 years after the last date of issue. |  |  |  |  |  |  |  |  |  |
| Reason(s) for issue: |  |  | Five-year update and updated to latest GHS requirements |  |  |  |  |  |  |
| Key abbreviations or acronyms used: |  |  | ADG Code - Australian Code for the Transport of Dangerous Goods by Road and Rail (7th edition) AICIS – Australian Industrial Chemicals Introduction Scheme (formerly NICNAS) AIIC - Australian Inventory of Industrial Chemicals APVMA – Agricultural Pesticides and Veterinary Medicines Australia GHS - Globally Harmonised System of Classification and Labelling of Chemicals (7th revised edition) 2017 IARC - International Agency for Research on Cancer Preparation of Safety Data Sheets for Hazardous Chemicals Code of Practice (June 2023) STEL - Short term exposure limit means the average airborne concentration of a substance calculated over a 15-minute period. The STEL should not be exceeded at any time during a normal eight hour working day. SUSMP - Standard for the Uniform Scheduling of Medicines & Poisons SWA - Safe Work Australia, formerly ASCC and NOHSC TGA – Therapeutic Goods Australia TWA - Time-weighted average means the average airborne concentration of a particular substance when calculated over an eight-hour working day, for a five-day working week. WHS – Workplace Health and Safety |  |  |  |  |  |  |

The physical values and properties described in this SDS are typical values based on scientific literature and material produced to date, and are believed to be reliable. The manufacturer, Sabakem Pty Ltd provides no warranties, either expressed or implied and assumes no responsibility for the accuracy or completeness of the data contained herein.  The information is supplied upon the condition that the persons receiving information will make their own determination as to the suitability for their purposes prior to use of this product.  Due care should be taken to ensure that the use of this product and its disposal is in compliance with all relevant Federal, State and Local Government regulations.

End of SDS