Section 1 - Identification of The Material and Supplier

| Sabakem Pty Ltd Suite 809, Level 8, 2 Queen Street | Phone: 03 9629 3979 www.sabakem.com Emergency: 1800 033 111Fax: 1300 242 436 |
| --- | --- |
| Melbourne VIC 3000 AUSTRALIA Emergency: 1800 033 111 111 |  |

Chemical nature:

Trade Name:

APVMA Code:

Product Use:

Creation Date:

This version issued:

Atrazine is a 1,3,5-triazine derivative.

Sabakem Atrazine 900WG Herbicide

67452

Agricultural herbicide for use as described on the product label.

August 2013

July 2026 and is valid for 5 years from this date.

Emergency telephone:	Poisons Informations Centre 13 11 26 (24 hours) from anywhere in Australia, (0800 764 766 in New Zealand)

Section 2 - Hazards Identification

Statement of Hazardous Nature

This product is classified as hazardous according to the Globally Harmonised System of Classification and Labelling of Chemicals (GHS) and Safe Work Australia criteria.

ADG Classification: Class 9: Environmentally Hazardous

Not subject to the provisions of the Australian Code for the Transport of Dangerous Goods by Road and Rail when transported by road or rail in; packages 500kg(L) or less; or IBCs (refer to SP AU01). However, if transported by Air or Sea, this provision does not apply, and the product is classed as Dangerous (Class 9 Environmentally Hazardous) by IATA and IMDG respectively. See details below and in Section 14 of this SDS.

Classification of the substance or mixture:

Specific target organ toxicity (repeated exposure) Category 2

Eye damage/irritation Category 2

Skin Sensitisation Category 1

The following health hazard categories fall outside the scope of the Workplace Health and Safety Regulations

Acute Aquatic Toxicity Category 1

Chronic Aquatic Toxicity Category 1

GHS Signal word: WARNING

HAZARD STATEMENT:

H317: May cause an allergic skin reaction.

H373: May cause damage to organs through prolonged or repeated exposure.

H410: Very toxic to aquatic life with long lasting effects.

PRECAUTIONARY STATEMENT(S):

PREVENTION

P261: Avoid breathing dust.

P272: Contaminated work clothing should not be allowed out of the workplace.  
P280: Wear protective gloves, protective clothing and eye or face protection.

RESPONSE

P314: Get medical advice or attention if you feel unwell.

P363: Wash contaminated clothing before reuse.

P301+P330+P331: IF SWALLOWED: Rinse mouth. Do NOT induce vomiting.

P302+P352: IF ON SKIN: Wash with plenty of soap and water.

P333+P313: If skin irritation or rash occurs: Get medical advice.

P337+P313: If eye irritation persists: Get medical advice.

P391: Collect spillage.

P370+P378: In case of fire, use carbon dioxide, dry chemical, foam, water fog. Water fog or fine spray is the preferred medium for large fires.

STORAGE

P403+P233: Store in a well-ventilated place. Keep container tightly closed.

DISPOSAL

P501: Dispose of contents and containers as specified on the registered label.

# Section 3 - Composition/Information on Ingredients

| Ingredients | CAS No | Conc (% w/w) |
| --- | --- | --- |
| Atrazine | 1912-24-9 | 90 |

Other components are not considered hazardous in this formulation and therefore are not required to be disclosed according to the WHS Regulations.

# Section 4 - First Aid Measures

General Information:

You should call The Poisons Information Centre if you feel that you may have been poisoned, burned or irritated by this product. The number is 13 1126 from anywhere in Australia (0800 764 766 in New Zealand) and is available at all times. Have this SDS with you when you call.

Inhalation: No first aid measures normally required. However, if inhalation has occurred, and irritation has  
developed, remove to fresh air and observe until recovered. If irritation becomes painful or persists more than about 30 minutes, seek medical advice.

Skin Contact: Gently brush away excess particles. Wash gently and thoroughly with water (use non-abrasive soap

if necessary) for 5 minutes or until chemical is removed.   Under running water, remove contaminated clothing, shoes and leather goods (e.g. watchbands and belts) and completely decontaminate them before reuse or discard. If irritation persists, repeat flushing and seek medical attention.

Eye Contact: Quickly and gently brush particles from eyes. No effects expected. If irritation does occur, flush  
contaminated eye(s) with lukewarm, gently flowing water for 5 minutes or until the product is removed. Obtain medical advice if irritation becomes painful or lasts more than a few minutes. Take special care if exposed person is wearing  
contact lenses.

Ingestion: If product is swallowed or gets in mouth, do NOT induce vomiting; wash mouth with water and give some water to drink. If symptoms develop, or if in doubt contact a Poisons Information Centre or a doctor.

First Aid Facilities: Eyewash and normal washroom facilities. Safety deluge showers should, if practical, be provided near to where this product is being used.

Major Health Hazards: Atrazine may be harmful to humans and to animals. It can be absorbed orally, dermally, and by inhalation. Symptoms of poisoning include abdominal pain, diarrhoea and vomiting, eye irritation, irritation of mucous membranes, and skin reactions. This product is a possible skin sensitiser.

Section 5 - Fire Fighting Measures

Fire and Explosion Hazards: The major hazard in fires is usually inhalation of heated and toxic or oxygen

deficient (or both), fire gases. There is little risk of an explosion from this product if commercial quantities are involved in a fire. Violent steam generation or eruption may occur upon application of direct water stream on hot liquids. This  
product, if scattered, may form flammable or explosive dust clouds in air.

Fire decomposition products from this product may be toxic if inhaled. Take appropriate protective measures.

Suitable Extinguishing Media: In case of fire, use carbon dioxide, dry chemical, foam, water fog. Water fog or fine spray is the preferred medium for large fires. Try to contain spills, minimise spillage entering drains or water courses.

Special Protective Equipment and Precautions for Fire Fighters: If a significant quantity of this product is involved in a fire, call the fire brigade. There is little danger of a violent reaction or explosion if significant quantities of this product are involved in a fire. Recommended personal protective equipment is full fire kit and breathing apparatus. Do not scatter spilled material with high pressure water jets.

Hazchem Code: 2Z (bulk transport only)

Section 6 - Accidental Release Measures

Environmental precautions: In the event of a major spill, prevent spillage from entering drains or water courses. Wear full protective clothing including eye/face protection. All skin areas should be covered. See below under Personal  
Protection regarding Australian Standards relating to personal protective equipment. Suitable materials for protective  
clothing include cotton, rubber, PVC. Eye/face protective equipment should comprise as a minimum, protective  
goggles. If there is a significant chance that dusts are likely to build up in cleanup area, we recommend that you use a suitable Dust Mask. Use a P1 mask, designed for use against mechanically generated particles e.g. silica & asbestos. Otherwise, not normally necessary.

Methods and materials for containment and cleaning up: Spills & Disposal: Stop leak if safe to do so, and contain spill. Because of the environmentally hazardous nature of this product, special  care should be taken to restrict release to waterways or drains. Sweep up and shovel or collect recoverable product into labelled containers for recycling or salvage, and dispose of promptly. Consider vacuuming if appropriate. Recycle containers wherever possible after careful cleaning. Refer to product label for specific instructions. After spills, wash area preventing runoff from entering drains. If a significant quantity of material enters drains, advise emergency services. Full details regarding disposal of used containers, spillage and unused material may be found on the label. If there is any conflict between this SDS and the label, instructions on the label prevail. Ensure legality of disposal by consulting regulations prior to disposal. Thoroughly launder protective clothing before storage or re-use. Advise laundry of nature of contamination when sending contaminated clothing to laundry.

Personal precautions, protective equipment and emergency procedures: All skin areas should be covered. Wear full protective chemically resistant clothing including eye/face protection, gauntlets and self-contained breathing apparatus. Suitable materials for protective clothing include PVC, Nitrile. Use impermeable gloves with care. Eye/face protective equipment should comprise, as a minimum, protective goggles. Usually, no respirator is necessary when using this product however, if there is a significant chance that vapours or mists are likely to build up in the cleanup area, we recommend that you use a suitable respirator. Refer to section 8 of this SDS for details of personal protective measures, and make sure that those measures are followed. Thoroughly launder protective clothing before storage or re-use. Advise laundry of nature of contamination when sending contaminated clothing to laundry.

Evacuate all non-essential personnel from affected area. Extinguish all sources of ignition. Avoid sparks and open flames. No smoking. If a significant quantity of material enters drains, advise emergency services.

Section 7 - Handling and Storage

Precautions for Safe Handling:  Keep exposure to this product to a minimum, and minimise the quantities kept in work areas. Check Section 8 of this SDS for details of personal protective measures, and make sure that those measures are followed. The measures detailed below under "Storage" should be followed during handling in order to minimise risks to persons using the product in the workplace. Also, avoid contact or contamination of product with incompatible materials listed in Section 10.

Conditions for Safe Storage:  This product is a Scheduled Poison. Observe all relevant regulations regarding sale, transport and storage of this schedule of poison. Check packaging - there may be further storage instructions on the label.

Section 8 - Exposure Controls and Personal Protection

Exposure Standards:

SWA Exposure Limits 	TWA (mg/m3) 	STEL (mg/m3)

Atrazine 	5 	not set

Engineering Controls: This product should only be used in a well-ventilated area. If natural ventilation is inadequate, use of a fan is suggested.

Respiratory Protection: If there is a significant chance that dusts are likely to build up in the area where this product is being used, we recommend that you use a suitable Dust Mask. Otherwise, not normally necessary. Safety deluge showers should, if practical, be provided near to where this product is being handled commercially.

Eye and Face Protection:   Eye protection such as protective glasses or goggles is recommended when this product is being used.

Skin Protection: Make sure that all skin areas are covered. Use PVC or rubber gloves. See Australian/New Zealand Standard AS/NZS 2161 for more information. When selecting gloves for use against certain chemicals, the degradation resistance, permeation rate and permeation breakthrough time should be considered. Occupational protective clothing (depending on conditions in which it has to be used, in particular as regards the period for which it is worn, which shall be determined on the basis of the seriousness of the risk, the frequency of exposure to the risk, the characteristics of the workstation of each worker and the performance of the protective clothing). See Australian/New Zealand Standard Occupational Protective Clothing:  AS/NZS 4501 and Occupational Protective Footwear: AS/NZS2210 for more information.

Section 9 - Physical and Chemical Properties:

Physical Description & colour:

Odour:

Boiling Point:

Freezing/Melting Point:

Volatiles:

Light beige to brown granules.

No data.

Not available.

Atrazine melts at 175°C

No specific data. Expected to be low at 100°C.

Vapour Pressure:

Vapour Density:

Specific Gravity:

Water Solubility:

pH:

Volatility:

Odour Threshold:   Evaporation Rate:

Coeff Oil/water Distribution: Viscosity:

Autoignition temp:

Negligible at normal ambient temperatures. Not applicable.

No data. Bulk density 0.40-0.45

Wettable.

No data.

Negligible at normal ambient temperatures. No data.

Not applicable.

No data

Not applicable.

500°C

Section 10 - Stability and Reactivity

Possibility of Hazardous Reactions:  This product is unlikely to react or decompose under normal storage conditions. However, if you have any doubts, contact the supplier for advice on shelf-life properties.

This product will not undergo polymerisation reactions.

Conditions to Avoid: Protect this product from light. Store in the closed original container in a dry, cool, well- 
ventilated area out of direct sunlight.

Incompatibilities: strong acids, strong bases, strong oxidising agents.

Fire Decomposition: Combustion forms carbon dioxide, and if incomplete, carbon monoxide and possibly smoke.  Water is also formed. May form nitrogen and its compounds, and under some circumstances, oxides of nitrogen.  
Occasionally hydrogen cyanide gas in reducing atmospheres. May form hydrogen chloride gas, other compounds of  
chlorine. Carbon monoxide poisoning produces headache, weakness, nausea, dizziness, confusion, dimness of  
vision, disturbance of judgment, and unconsciousness followed by coma and death.

Section 11 - Toxicological Information

Acute toxicity: Atrazine may be harmful to humans and to animals. It can be absorbed orally, dermally, and by  
inhalation.

Following is the acute toxicity data available for the active constituent Atrazine:

Acute oral toxicity - LD50 (Rat): 3090 mg/kg

Acute dermal toxicity - LD50 (Rat) > 3000 mg/kg

Acute inhalation toxicity - LD50 (Rat) > 5.2 mg/L/4h

Skin Corrosion/Irritation: Classified as a potential sensitiser by skin contact. Exposure to a skin sensitiser, once  
sensitisation has occurred, may manifest itself as skin rash or inflammation, and in some individuals this reaction can be severe. In addition product may be irritating, but is unlikely to cause anything more than mild transient discomfort.

Serious Eye Damage/Irritation: Product may cause eye irritation, but is unlikely to cause anything more than mild  
transient discomfort.

Respiratory or Skin Sensitisation: May cause an allergic skin reaction.

Persons sensitised to Atrazine should avoid contact with this product.

Germ Cell Mutagenicity: The weight of evidence from more than 50 studies indicates that Atrazine is not mutagenic.

Carcinogenicity: Available data regarding Atrazine's carcinogenic potential are inconclusive.

Reproductive Toxicity: Based on classification principles, the classification criteria are not met.

Specific Target Organ Toxicity (STOT)—single exposure: Lethal doses of Atrazine in test animals have caused congestion and/or hemorrhaging to the lungs, kidneys, liver, spleen, brain, and heart.

Specific Target Organ Toxicity (STOT)—repeated exposure: Long-term consumption of high levels of Atrazine has caused tremors, changes in organ weights, and damage to the liver and heart.

Aspiration Hazard: Based on classification principles, the classification criteria are not met.

Chronic Health Effects: No data available.

Additional toxicological information:

The ADI for Atrazine is set at 0.005mg/kg/day and the corresponding NOEL is set at 0.5mg/kg/day. Data from Australian ADI List, June 2014. ADI means Acceptable Daily Intake; NOEL means No-observable-effect-level.

Section 12 - Ecological Information

Ecotoxicity: Very toxic to aquatic organisms, may cause long-term adverse effects to the aquatic environment.

Persistence and Degradability: Atrazine is highly persistent in soil. Chemical hydrolysis, followed by degradation by soil microorganisms, accounts for most of the breakdown of Atrazine. Hydrolysis is rapid in acidic or basic environments, but is slower at neutral pHs. Addition of organic material increases the rate of hydrolysis.

Atrazine is moderately soluble in water. Chemical hydrolysis, followed by biodegradation, may be the most important route of disappearance from aquatic environments. Hydrolysis is rapid under acidic or basic conditions, but is slower at neutral pHs. Atrazine is not expected to strongly adsorb to sediments.

Bioaccumulative Potential: Atrazine is absorbed by plants mainly through the roots, but also through the foliage.  
Once absorbed, it is translocated upward and accumulates in the growing tips and the new leaves of the plant. In  
susceptible plant species, Atrazine inhibits photosynthesis. In tolerant plants, it is metabolized.

Mobility in Soil: Atrazine exhibits mid to high mobility in soil.

Section 13 - Disposal Considerations

Disposal: Special help is available for the disposal of Agricultural Chemicals. The product label will give general advice regarding disposal of small quantities, and how to cleanse containers. However, for help with the collection of unwanted rural chemicals, contact ChemClear 1800 008 182 http://www.chemclear.com.au/ and for help with the disposal of empty drums, contact DrumMuster http://www.drummuster.com.au/ where you will find contact details for your area.

Section 14 - Transport Information

Road and Rail Transport

Australian Special Provisions; AU01: Environmentally Hazardous Substances meeting the description of UN 3077 are not subject to this Code when transported by road or rail in;

(a) packaging’s that do not incorporate a receptacle exceeding 500 Kg (L); or

(b) IBCs.

Marine Transport:

Classified as Dangerous Goods by the criteria of the International Maritime Dangerous Goods Code (IMDG Code) for transport by sea; DANGEROUS GOODS.

| UN Number: | 3077 |
| --- | --- |
| Proper Shipping Name or Technical Name: | ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S (CONTAINS ATRAZINE) |
| Transport Hazard Class: | 9 |
| Packaging Group: | III |
| IMDG EMS Fire: | F - A |
| IMDG EMS Spill: | S - F |
| Environmental hazards: | Yes. |
| Additional Information: | The marine pollutant mark is not required when transported in sizes of ≤ 5 L or ≤ 5 kg. |

Air Transport:

IATA provision SP A197: Environmentally Hazardous Substances meeting the description of UN 3077 or UN 3082 are not subject to this Code when transported air in; packages that have inner packages (plastic bottles, glass bottles, plastic bags) of 5 L for UN3082 and 5 kg for UN3077 or less.

Section 15 - Regulatory Information

APVMA Approval no.: 67452

Poison schedule (SUSMP): Schedule 5

AICIS: All the constituents of this material are either listed on the Australian Inventory of Industrial Chemicals (AIIC), not required due the nature of the chemical as they are excluded as an industrial chemical or have been assessed under the Industrial Chemicals Act 1989 as amended.

Section 16 - Other Information

This SDS contains only safety-related information. For other data see product literature.

Issue Date: July 2026

Reason(s) for issue: Five-year update and updated to latest GHS requirements.

Key abbreviations or acronyms:

ADG Code - Australian Code for the Transport of Dangerous Goods by Road and Rail (7th edition)

AICIS – Australian Industrial Chemicals Introduction Scheme (formerly NICNAS)

AIIC - Australian Inventory of Industrial Chemicals

APVMA – Agricultural Pesticides and Veterinary Medicines Australia

CAS number - Chemical Abstracts Service Registry Number

GHS - Globally Harmonised System of Classification and Labelling of Chemicals (7th revised edition) 2017

Hazchem Code - Emergency action code of numbers and letters that provide information to emergency services especially firefighters

IARC - International Agency for Research on Cancer

Preparation of Safety Data Sheets for Hazardous Chemicals Code of Practice (June 2023)

STEL - Short term exposure limit means the average airborne concentration of a substance calculated over a 15 minute period. The STEL should not be exceeded at any time during a normal eight hour working day.

SUSMP - Standard for the Uniform Scheduling of Medicines & Poisons

SWA - Safe Work Australia, formerly ASCC and NOHSC

TGA – Therapeutic Goods Australia

TWA - Time-weighted average means the average airborne concentration of a particular substance when calculated over an eight-hour working day, for a five-day working week.

UN Number - United Nations Number

WHS – Workplace Health and Safety

THIS SDS SUMMARISES OUR BEST KNOWLEDGE OF THE HEALTH AND SAFETY HAZARD INFORMATION OF THE PRODUCT AND HOW TO SAFELY HANDLE AND USE THE PRODUCT IN THE WORKPLACE. EACH USER MUST REVIEW THIS SDS IN THE CONTEXT OF HOW THE PRODUCT WILL BE HANDLED AND USED IN THE WORKPLACE. IF CLARIFICATION OR FURTHER INFORMATION IS NEEDED TO ENSURE THAT AN APPROPRIATE RISK ASSESSMENT CAN BE MADE, THE USER SHOULD CONTACT THIS COMPANY SO WE CAN ATTEMPT TO OBTAIN ADDITIONAL INFORMATION FROM OUR SUPPLIERS OUR RESPONSIBILITY FOR PRODUCTS SOLD IS SUBJECT TO OUR STANDARD TERMS AND CONDITIONS, A COPY OF WHICH IS SENT TO OUR CUSTOMERS AND IS ALSO AVAILABLE ON REQUEST.

Please read all labels carefully before using product.

This SDS is prepared in accord with the SWA document “Preparation of Safety Data Sheets for Hazardous

Chemicals - Code of Practice” (June 2023) and GHS Revision 7

Copyright © Sabakem Pty Ltd.